e-Yantra Robotics Competition - 2018

Theme and Implementation Analysis – Thirsty Crow

<Team ID>

| Team leader name | Akash Pandey |
| --- | --- |
| College | Maharaja Agrasen Institute Of Technology |
| Email | aakashpandey33@gmail.com |
| Date | 20-01-2019 |

Scope

Q1 a. State the scope of the theme assigned to you. 							(5)

< Teams should briefly explain in their own words the theme assigned. What in your opinion is the purpose of such an application? You may use figures / diagrams to support your answer.

Answer format: Text - limit: 100 words. >

The theme assigned to us is thirsty crow, and it recreates the story of well Known thirsty crow and pitcher by using advance technology like bots and augmented reality.

In this theme, a bot on behalf of crow is deployed to collect pebbles from different locations in the arena in minimum time and drop it at a particular location which act as a pitcher. At same time, on a virtual screen whole story plays with the help of AR and projections of 3D models.

Purpose of such an application is to build boots like this which can solve many real life problems in very less time. For example.

These bots can detect metal in garbage and separate it out which can be reused, etc.

These robots can be very useful in terms of transportation in big factories

These robot can also serve as a waiter in restaurants , for providing food(contained in proper food table)

These bots can create immense difference in helping and securing society
Testing your knowledge (theme and rulebook analysis)

Q2. a) What is considered a correct pebble pick-up?						  (2.5)

b) What is considered a correct pebble drop?							  (2.5)

< Explain for both, the arena traversal part and the augmented reality part. >

- When a magnetic pebble is picked up by the robot, 3D model of pebbles projected on the Pebble AR Object should reflect the change by showing the 3Dmodel to be diminished and once picked should not drop

- When pebble is dropped by the robot under the Water Pitcher AR_Object, 3D model of water pitcher should reflect the same by                    indicating rise in water level, once dropped it should not stick immediately.

Mechanism

Q3. Explain the mechanism that you would use to pick and drop the pebbles.  (10)

< Explain how you would mount the electromagnet on the robot and what mechanism you would use for the electromagnetic pick & drop system. >

We will be using the magnetic properties of the electromagnet in order to pick up and drop the pebbles. The robot has a magnet attached to the hand which can be controlled programmatically. A servo motor is connected to the fulcrum of the hand of the robot. The magnet is connected at the outer tip of the hand and the magnet is controlled by a signal to activate and deactivate. The robot has a bag which stores the collected object.

The robot moves forward towards the object. As it reaches closer to the object, the robot is halted at a distance of it’s hand from the object. Upon reaching, a signal is passed to the servo which then rotates and brings the hand from upward position to forward position. After a small delay is applied, a signal is sent to the magnet for activation.

As the magnet activates, the object beneath is attracted to it and sticks to the hand. Then a signal is passed to the servo for rotating from forward position to backward position in counter clockwise direction. When the rotation is complete, the arm should be just above the collecting bag. A signal is then sent to the magnet for deactivation.

As the magnet deactivates the object is detached from the hand and drops inside the bag. Now the hand can either be moved to forward position or can be kept at the same position. Now if any other object is remaining in the arena, the robot will again follow the same process, otherwise it moves to the finish position.

Algorithm Analysis

Q4. Draw a flowchart illustrating the algorithm you propose to use for theme implementation.													         (10)

< The flowchart should elaborate on every possible function that you will be using for completing all the tasks in the assigned theme.

Follow the standard pictorial representation used to draw the flowchart. >

start

Root is set at local node and then forward to the tentative list

Is tentative list empty?

no

NO

YES

From the tentative list move the node with smaller weight (shortest path) to the permanent list

Add each unprocessed neighbour node of the last moved node to tentative list if not there . If the node with larger cost then replace it with other new

Q5. What kind of path planning algorithm will you use in order to navigate your robot inside the arena? 													  (10)

< Explain the logic behind the algorithm and the reason for your choice if any. You can use a pseudo-code to help elucidate your answer. >

The arena consists of hexagonal mazes and some mazes contain the Arocu markers. So we can imagine the arena as combination of edges and nodes where each node can have a maximum of 6 edges. So the mechanism to collect the pebbles would be to follow Dijkstra’s method and reach the nearest Arocu marker following the shortest path.

Using the Dijkstra algorithm, it is possible to determine the shortest distance (or the least effort / lowest cost) between a start node and any other node in a graph. The idea of the algorithm is to continiously calculate the shortest distance beginning from a starting point, and to exclude longer distances when making an update. It consists of the following steps:

- Initialization of all nodes with distance "infinite"; initialization of the starting node with 0

- Marking of the distance of the starting node as permanent, all other distances as temporarily.

- Setting of starting node as active.

- Calculation of the temporary distances of all neighbour nodes of the active node by summing up its distance with the weights of the edges.

- If such a calculated distance of a node is smaller as the current one, update the distance and set the current node as antecessor. This step is also called update and is Dijkstra's central idea.

- Setting of the node with the minimal temporary distance as active. Mark its distance as permanent.

- Repeating of steps 4 to 7 until there aren't any nodes left with a permanent distance, which neighbours still have temporary distances.

| 1: | function Dijkstra(Graph, source): |
| --- | --- |
| 2: | for each vertex v in Graph: | // Initialization |
| 3: | dist[v] := infinity | // initial distance from source to vertex v is set to infinite |
| 4: | previous[v] := undefined | // Previous node in optimal path from source |
| 5: | dist[source] := 0 | // Distance from source to source |
| 6: | Q := the set of all nodes in Graph | // all nodes in the graph are unoptimized - thus are in Q |
| 7: | while Q is not empty: | // main loop |
| 8: | u := node in Q with smallest dist[ ] |  |
| 9: | remove u from Q |  |
| 10: | for each neighbor v of u: | // where v has not yet been removed from Q. |
| 11: | alt := dist[u] + dist_between(u, v) |  |
| 12: | if alt < dist[v] | // Relax (u,v) |
| 13: | dist[v] := alt |  |
| 14: | previous[v] := u |  |
| 15: | return previous[ ] |  |

Challenges

Q6. What are the major challenges that you can anticipate in addressing this theme and how do you propose to tackle them? 							  (10)

< Answer format: Bullet points

1. Challenge 1

2. Challenge 2

3. Challenge 3, etc. >

1. making the arm with a simple structure but does the same work as done by a complex one, we achieved it by attaching a piece of wood in the form of simple cantilever beam

2. Task 1.2 (image processing) was really a hard task to understand , but we did it finally with the help of instructors

3. wires were untidy when we complete the whole circuit we tackled this problem by combining some wires together.

4. our white line sensor is not working and for that we have asked our instructor on piazza about the problem

5. Aruco detection , was a major challenge  . we tackled it by gaining some knowledge from instructors.

6. Doing connections , this was very hactic taks . But we tackle it by hardwork

7. Doing soldering was a great challenge and with a lot practice we tackled it.

8. Learning a new programming language ( python) including it’s libraries opencv and opengl , we learned it with resources provided by team-eyantra

9. Finding Terminal blocks , Jumpers(F to F) , Nuts in the market as they are not easily available in near by shops , we tacked this issue for jumpers by making F to F by soldering two M to F wires

10. Finding acrylic sheet for bot base , but cutting acrylic sheet was too costly so we used wood base for bot.